`timescale 1ns/1ps

// ============================================================================

// 1. PROGRAM COUNTER

// ============================================================================

module program_counter (

input  logic        clk,

input  logic        reset,

input  logic [31:0] next_pc,

output logic [31:0] pc

);

always_ff @(posedge clk or posedge reset) begin

if (reset)

pc <= 32'b0;

else

pc <= next_pc;

end

endmodule

// ============================================================================

// 2. INSTRUCTION MEMORY

// ============================================================================

module instruction_memory (

input  logic [31:0] address,

output logic [31:0] instruction

);

logic [31:0] memory [0:255];

initial begin

integer i;

for (i = 0; i < 256; i = i + 1)

memory[i] = 32'b0;

// Example RV32I Test Program:

memory[0] = 32'h00a00093; // ADDI x1, x0, 10

memory[1] = 32'h01400113; // ADDI x2, x0, 20

memory[2] = 32'h002081b3; // ADD  x3, x1, x2

memory[3] = 32'h00000013; // NOP (Triggers sleep mode)

end

always_comb begin

instruction = memory[address[9:2]];

end

endmodule

// ============================================================================

// 3. DECODER

// ============================================================================

module decoder (

input  logic [31:0] instruction,

output logic        reg_write,

output logic        mem_read,

output logic        mem_write,

output logic        mem_to_reg,

output logic        alu_src,

output logic        branch,

output logic        branch_ne,

output logic        jump,

output logic        jalr,

output logic [3:0]  alu_control

);

logic [6:0] opcode;

logic [2:0] funct3;

logic [7:0] funct7;

assign opcode = instruction[6:0];

assign funct3 = instruction[14:12];

assign funct7 = instruction[31:25];

always_comb begin

reg_write   = 1'b0;

mem_read    = 1'b0;

mem_write   = 1'b0;

mem_to_reg  = 1'b0;

alu_src     = 1'b0;

branch      = 1'b0;

branch_ne   = 1'b0;

jump        = 1'b0;

jalr        = 1'b0;

alu_control = 4'b0000;

case (opcode)

7'b0110011: begin // R-Type

reg_write = 1'b1;

case (funct3)

3'b000: alu_control = (funct7 == 7'b0100000) ? 4'b0001 : 4'b0000;

3'b111: alu_control = 4'b0010;

3'b110: alu_control = 4'b0011;

3'b100: alu_control = 4'b0100;

3'b001: alu_control = 4'b0101;

3'b101: alu_control = 4'b0110;

default: alu_control = 4'b0000;

endcase

end

7'b0010011: begin // I-Type Arithmetic

reg_write = 1'b1;

alu_src   = 1'b1;

case (funct3)

3'b000: alu_control = 4'b0000;

3'b111: alu_control = 4'b0010;

3'b110: alu_control = 4'b0011;

default: alu_control = 4'b0000;

endcase

end

7'b0000011: begin // Load (LW)

reg_write  = 1'b1;

mem_read   = 1'b1;

mem_to_reg = 1'b1;

alu_src    = 1'b1;

alu_control= 4'b0000;

end

7'b0100011: begin // Store (SW)

mem_write  = 1'b1;

alu_src    = 1'b1;

alu_control= 4'b0000;

end

7'b1100011: begin // Branch (BEQ, BNE)

if (funct3 == 3'b000) branch    = 1'b1;

if (funct3 == 3'b001) branch_ne = 1'b1;

alu_control = 4'b0001;

end

7'b1101111: begin // JAL

reg_write = 1'b1;

jump      = 1'b1;

end

7'b1100111: begin // JALR

reg_write = 1'b1;

jalr      = 1'b1;

alu_src   = 1'b1;

end

default: ;

endcase

end

endmodule

// ============================================================================

// 4. REGISTER FILE

// ============================================================================

module register_file (

input  logic        clk,

input  logic        reset,

input  logic [4:0]  rs1,

input  logic [4:0]  rs2,

input  logic [4:0]  rd,

input  logic [31:0] write_data,

input  logic        reg_write,

output logic [31:0] read_data1,

output logic [31:0] read_data2

);

logic [31:0] registers [0:31];

always_ff @(posedge clk or posedge reset) begin

if (reset) begin

integer i;

for (i = 0; i < 32; i = i + 1)

registers[i] <= 32'b0;

end

else if (reg_write && (rd != 5'b0)) begin

registers[rd] <= write_data;

end

end

assign read_data1 = (rs1 == 5'b0) ? 32'b0 : registers[rs1];

assign read_data2 = (rs2 == 5'b0) ? 32'b0 : registers[rs2];

endmodule

// ============================================================================

// 5. IMMEDIATE GENERATOR

// ============================================================================

module immediate_generator (

input  logic [31:0] instruction,

output logic [31:0] immediate

);

logic [6:0] opcode;

assign opcode = instruction[6:0];

always_comb begin

case (opcode)

7'b0010011, 7'b0000011, 7'b1100111:

immediate = {{20{instruction[31]}}, instruction[31:20]};

7'b0100011:

immediate = {{20{instruction[31]}}, instruction[31:25], instruction[11:7]};

7'b1100011:

immediate = {{20{instruction[31]}}, instruction[7], instruction[30:25], instruction[11:8], 1'b0};

7'b1101111:

immediate = {{12{instruction[31]}}, instruction[19:12], instruction[20], instruction[30:21], 1'b0};

default:

immediate = 32'b0;

endcase

end

endmodule

// ============================================================================

// 6. LOW-POWER CONTROLLER

// ============================================================================

module low_power_controller (

input  logic reg_write,

input  logic mem_read,

input  logic mem_write,

input  logic branch,

input  logic jump,

output logic operand_isolation,

output logic sleep_mode

);

always_comb begin

if (!reg_write && !mem_read && !mem_write && !branch && !jump) begin

operand_isolation = 1'b1;

sleep_mode        = 1'b1;

end

else begin

operand_isolation = 1'b0;

sleep_mode        = 1'b0;

end

end

endmodule

// ============================================================================

// 7. ARITHMETIC LOGIC UNIT (ALU)

// ============================================================================

module alu (

input  logic [31:0] operand_a,

input  logic [31:0] operand_b,

input  logic [3:0]  alu_control,

input  logic        operand_isolation,

output logic [31:0] result,

output logic        zero

);

logic [31:0] a_in;

logic [31:0] b_in;

assign a_in = operand_isolation ? 32'b0 : operand_a;

assign b_in = operand_isolation ? 32'b0 : operand_b;

always_comb begin

if (operand_isolation) begin

result = 32'b0;

end

else begin

case (alu_control)

4'b0000: result = a_in + b_in;

4'b0001: result = a_in - b_in;

4'b0010: result = a_in & b_in;

4'b0011: result = a_in | b_in;

4'b0100: result = a_in ^ b_in;

4'b0101: result = a_in << b_in[4:0];

4'b0110: result = a_in >> b_in[4:0];

default: result = 32'b0;

endcase

end

end

assign zero = (result == 32'b0);

endmodule

// ============================================================================

// 8. DATA MEMORY

// ============================================================================

module data_memory (

input  logic        clk,

input  logic        mem_read,

input  logic        mem_write,

input  logic [31:0] address,

input  logic [31:0] write_data,

output logic [31:0] read_data

);

logic [31:0] memory [0:255];

initial begin

integer i;

for (i = 0; i < 256; i = i + 1)

memory[i] = 32'b0;

end

always_comb begin

if (mem_read)

read_data = memory[address[9:2]];

else

read_data = 32'b0;

end

always_ff @(posedge clk) begin

if (mem_write)

memory[address[9:2]] <= write_data;

end

endmodule

// ============================================================================

// 9. WRITEBACK MULTIPLEXER

// ============================================================================

module writeback_mux (

input  logic [31:0] alu_result,

input  logic [31:0] memory_data,

input  logic [31:0] pc_plus_4,

input  logic        mem_to_reg,

input  logic        jump,

output logic [31:0] writeback_data

);

always_comb begin

if (mem_to_reg)

writeback_data = memory_data;

else if (jump)

writeback_data = pc_plus_4;

else

writeback_data = alu_result;

end

endmodule

// ============================================================================

// 10. BRANCH UNIT

// ============================================================================

module branch_unit (

input  logic        branch,

input  logic        branch_ne,

input  logic        jump,

input  logic        jalr,

input  logic [31:0] pc,

input  logic [31:0] rs1_data,

input  logic [31:0] rs2_data,

input  logic [31:0] immediate,

output logic [31:0] next_pc

);

logic equal;

assign equal = (rs1_data == rs2_data);

always_comb begin

if (jalr)

next_pc = (rs1_data + immediate) & ~32'd1;

else if (jump)

next_pc = pc + immediate;

else if (branch && equal)

next_pc = pc + immediate;

else if (branch_ne && !equal)

next_pc = pc + immediate;

else

next_pc = pc + 32'd4;

end

endmodule

// ============================================================================

// 11. TOP-LEVEL PROCESSOR CORE

// ============================================================================

module riscv_top (

input  logic        clk,

input  logic        reset,

output logic [31:0] pc_out,

output logic [31:0] alu_out,

output logic [31:0] writeback_data,

output logic        sleep_mode

);

logic [31:0] pc, next_pc, instruction;

logic [4:0]  rs1, rs2, rd;

logic [31:0] rs1_data, rs2_data, immediate;

logic        reg_write, mem_read, mem_write, mem_to_reg;

logic        alu_src, branch, branch_ne, jump, jalr;

logic [3:0]  alu_control;

logic [31:0] alu_operand_b, alu_result, memory_data, wb_data;

logic        operand_isolation;

assign rs1 = instruction[19:15];

assign rs2 = instruction[24:20];

assign rd  = instruction[11:7];

assign alu_operand_b  = alu_src ? immediate : rs2_data;

assign pc_out         = pc;

assign alu_out        = alu_result;

assign writeback_data = wb_data;

program_counter PC (

.clk(clk),

.reset(reset),

.next_pc(next_pc),

.pc(pc)

);

instruction_memory IMEM (

.address(pc),

.instruction(instruction)

);

decoder DECODER (

.instruction(instruction),

.reg_write(reg_write),

.mem_read(mem_read),

.mem_write(mem_write),

.mem_to_reg(mem_to_reg),

.alu_src(alu_src),

.branch(branch),

.branch_ne(branch_ne),

.jump(jump),

.jalr(jalr),

.alu_control(alu_control)

);

register_file REGFILE (

.clk(clk),

.reset(reset),

.rs1(rs1),

.rs2(rs2),

.rd(rd),

.write_data(wb_data),

.reg_write(reg_write),

.read_data1(rs1_data),

.read_data2(rs2_data)

);

immediate_generator IMM_GEN (

.instruction(instruction),

.immediate(immediate)

);

low_power_controller LOW_POWER (

.reg_write(reg_write),

.mem_read(mem_read),

.mem_write(mem_write),

.branch(branch),

.jump(jump),

.operand_isolation(operand_isolation),

.sleep_mode(sleep_mode)

);

alu ALU (

.operand_a(rs1_data),

.operand_b(alu_operand_b),

.alu_control(alu_control),

.operand_isolation(operand_isolation),

.result(alu_result),

.zero()

);

data_memory DMEM (

.clk(clk),

.mem_read(mem_read),

.mem_write(mem_write),

.address(alu_result),

.write_data(rs2_data),

.read_data(memory_data)

);

writeback_mux WB_MUX (

.alu_result(alu_result),

.memory_data(memory_data),

.pc_plus_4(pc + 32'd4),

.mem_to_reg(mem_to_reg),

.jump(jump),

.writeback_data(wb_data)

);

branch_unit BRANCH (

.branch(branch),

.branch_ne(branch_ne),

.jump(jump),

.jalr(jalr),

.pc(pc),

.rs1_data(rs1_data),

.rs2_data(rs2_data),

.immediate(immediate),

.next_pc(next_pc)

);

endmodule